ALTERATION PROJECT PROCEDURES

PART 1 - GENERAL

1.1	SUMMARY

A.	This Section specifies administrative and procedural requirements for Alteration or Modernization Projects for acceptance of existing site conditions, selective demolition, cutting and patching of existing buildings and site improvements, removal and reinstallation of existing materials, wiring and equipment and interface with existing construction.

B.	Refer to other Sections for specific requirements and limitations applicable to Alteration or Modernization projects

C.	Requirements of this Section apply to Sections in Divisions 2 through 16.

1.2	RELATED SECTIONS

A.	Section 01 01 00 - Summary of work.

B.	Section 01 73 20 - Cutting and Patching

1.3	ALTERATION PROJECTS GENERAL PROCEDURES

A.	Alteration/Modernization projects require that the contractor may need to demolish, cut, alter, expose, modify, repair, replace, reconstruct, patch, reroute, or other construction procedures to interface new construction into existing construction.

B.	The Drawings and specifications are not intended to show in detail all Alteration Project Procedures for interface of new construction into existing construction. It is the responsibility of the Contractor to include in the Contract Price Allowances for such Alteration Procedures.

1.4	QUALITY ASSURANCE

A.	Matching existing Construction: On Alteration\Modernization projects new materials are to match existing materials for patching and extending work.

B.	Determine type and quality of existing materials by inspection and testing. Existing construction shall be used as a standard of quality for new construction unless noted or specified otherwise.

### PART 2 - PRODUCTS

2.1	MATERIALS

A.	Use materials that are identical to existing materials.  If identical materials are not available or cannot be used where exposed surfaces are involved, use materials that match existing adjacent surfaces to the fullest extent possible with regard to visual effect.  Use materials whose installed performance will equal or surpass that of existing materials.

### PART 3 - EXECUTION

3.1	EXAMINATION

A.	Verify that selective demolition is complete and areas are ready for installation of new work.

3.2	PREPARATION

A.	Cut, move, or remove items as necessary for access to alteration and renovation work. Replace and restore prior to completion.

B.	Remove unsuitable material not marked for salvage, such as rotted wood, corroded metals, and deteriorated construction. Replace materials as specified for the affected finish material.

C.	Remove debris and abandoned items from area and from concealed spaces.

D.	Remove surface finishes to provide for proper installation of new work.

E.	Temporarily close openings in exterior surfaces to protect existing improvement from weather, temperature and humidity during construction of new work.

3.3	INSTALLATION

A.	Coordinate work of Alteration/Modernization to expedite completion and to accommodate School occupancy of the facility.

B.	Remove, cut and patch in a manner to minimize damage to existing facilities and to provide a means of restoring materials and finishes to original conditions.

C.	Refinish visible existing surfaces to remain in Alteration/Modernization areas to specified condition for each material, with a neat transition to adjacent finishes.

D.	In addition to the specified new equipment, fixtures, wiring, conduit, materials, etc. bring existing systems to full operational conditions before Alteration/Modernization work is completed.

E.	Patch, repair and refinish work that was damaged during mechanical, electrical and other modernization work.

3.4	TRANSITIONS

A.	Where the removal or addition of walls, ceilings and finishes abuts existing construction, construct a smooth and even transition. Patch new work to existing to match adjacent work in texture and appearance.

B.	When existing surfaces are cut so that a smooth transition with new construction is not possible, terminate existing surface along a straight line at a natural line of division, such as a corner change in finish or a joint. Replace existing finish as required for a smooth transition.

C.	Trim bottom of existing doors as required to clear new floor finish.

3.5	CONSTRUCTION INTERFERENCE

A.	Where existing construction interferes with new construction, such as pipes, conduit, junction boxes, and other existing construction that may be in a location that is not compatible with new construction, contractor is to relocate, move, provide replacement or otherwise remove the construction interference.

B.	Contractor is to field verify existing conditions and is not to reply on Existing Record Drawings provided by the District. Contractor is not to rely on any verbal instructions or verbal locations given by  District Personnel unless given or stated in writing. Existing Record drawings if provided are for information only and may not indicate the exact existing construction.

3.6	REPAIR OF DAMAGED SURFACES

A.	Where removal of partitions, ceilings, walls or finishes results in adjacent spaces becoming damaged, rework floors, walls and ceilings to provide for a smooth plane without break, steps, or bulkheads.

B.	Patch or replace portions of existing surfaces which are damaged, lifted, discolored, or showing other imperfections. Repair substrate prior to patching finish.

3.7	FINISHES

A.	Finish surfaces as specified in individual Product sections.

B.	Finish patches to produce uniform finish and texture over the entire area. When finish cannot be matched, refinish entire surface to nearest joint corner or intersection.

# END OF SECTION