CUTTING AND PATCHING

### PART 1 - GENERAL

1.1	SUMMARY

A.	This Section specifies administrative and procedural requirements for cutting and patching, and interface of new work into existing construction and with work being performed under other contracts provided by the School District.

B.	Refer to other Sections for specific requirements and limitations applicable to cutting and patching individual parts of the work.

1.	Requirements of this Section apply to Sections in Divisions 2 through 16.

1.2	RELATED SECTIONS

- Section 01 01 00 - Summary of work (Scope of Work).

1.3	SUBMITTALS

A.	Cutting and Patching Proposal:  Approval of procedures for cutting and patching is required before proceeding, submit a proposal describing procedures well in advance of the time cutting and patching will be performed and request approval to proceed.  Include the following information, as applicable, in the proposal:

1.	Describe the extent of cutting and patching required and how it is to be performed.

2.	Indicate dates when cutting and patching is to be performed.

3.	List utilities that will be disturbed or affected, including those that will be relocated and those that will be temporarily out-of-service.  Indicate how long service will be disrupted.

4.	Refer to Structural Drawing for locations where cutting and patching involves addition of reinforcement to structural elements. Do not damage or weaken existing structural elements.

5.	Approval by the Architect/Engineer to proceed with cutting and patching does not waive the Architect's right to later require complete removal and replacement of a part of the work found to be unsatisfactory.

1.4	QUALITY ASSURANCE

A.	Visual Requirements:  Do not cut and patch construction exposed on the exterior or in occupied spaces, in a manner that would, in the Architect's/Engineer’s opinion, reduce the building's aesthetic qualities, or result in visual evidence of cutting and patching.  Remove and replace work cut and patched in a visually unsatisfactory manner.

### PART 2 - PRODUCTS

2.1	MATERIALS

A.	Use materials that are identical to existing materials.  If identical materials are not available or cannot be used where exposed surfaces are involved, use materials that match existing adjacent surfaces to the fullest extent possible with regard to visual effect.  Use materials whose installed performance will equal or surpass that of existing materials.

### PART 3 - EXECUTION

3.1	INSPECTION

A.	Before cutting existing surfaces, examine surfaces to be cut and patched and conditions under which cutting and patching is to be performed.  Take corrective action before proceeding, if unsafe or unsatisfactory conditions are encountered.

3.2	PREPARATION

A.	Temporary Support:  Provide temporary support of work to be cut.

B.	Protection:  Protect existing construction during cutting and patching to prevent damage.  Provide protection from adverse weather conditions for portions of the Project that might be exposed during cutting and patching operations.

C.	Avoid interference with use of adjoining areas or interruption of free passage to adjoining areas.

D.	Take all precautions necessary to avoid cutting existing pipe, electrical wire and conduit or ductwork serving the building.

3.3	PERFORMANCE

A.	General:  Employ skilled workmen to perform cutting and patching. Proceed with cutting and patching at the earliest feasible time and complete without delay.

1.	Cut existing construction to provide for installation of other components or performance of other construction activities and the subsequent fitting and patching required to restore surfaces to their original condition.

2.	Where patching occurs in a smooth painted surface, extend final paint coat over entire surface containing the patch, after the patched area has received primer and second coat.

3.	Cut, patch, point-up and repair plaster to accommodate other construction and to repair cracks, dents and imperfections.

4.	Cut, patch, restore and repair all gypsum board wall and ceiling surfaces where new pipes, equipment, clocks, switches, conduit, ducts and any new construction items that would damage or cut existing surfaces.

5.	Cut patch and repair existing concrete and asphalt paving where new utility lines are installed across existing paving and under existing concrete floor slabs. Site verify extent of cutting and patching required. All existing site improvements may not be indicated on the site plan and floor plans.

6.	Cut existing walls, floors, ceilings and roofs or other parts of building structure to accommodate new ducts, conduits and piping, patch and repair existing.

7.	Patch existing floors, walls, roofs and ceilings where existing ducts, conduit, equipment, water, gas, sewer, windows, doors etc. that are not used or removed and are not to be replaced. This is considered part of required general patching and is part of the contract and will not be shown in detail on the Contract drawings. Field verify with existing site and building construction for patching required.

3.4	CLEANING

A.	Thoroughly clean areas and spaces where cutting and patching is performed or used as access.  Remove completely paint, mortar, oils, putty and items of similar nature.  Thoroughly clean piping, conduit and similar features before painting or other finishing is applied.  Restore damaged areas to their original condition.

# END OF SECTION